Laser Diode Driver PCB SG-LD1:

----------------------------

Board type: Single sided.  Plated through-holes are optional.

Board size: 1.7" L (X) x 1.15" W (Y).

Suggested board thickness: .050" to .062".

Design rules: Signal - 10 mil traces, Power - 24 mil traces, 10 mil clearance

minimum.

Files:

SGLD1.SLK - Top silkscreen (part designations, etc.) (Gerber).

SGLD1.TOP - Top copper pads (optional using plated through-holes) (Gerber).

SGLD1.BOT - Bottom (solder side) copper traces and pads (Gerber).

SGLD1.BSM - Bottom (solder side) soldermask (optional) (Gerber).

SGLD1.DRL - Drill location plot (Gerber).

SGLD1.NCD - Drill control file.

SGLD1.REP - Photo/laserplotter aperture information, parts list, statistics.

SGLD1.PCB, SGLD1.INI, SGLD1.DFN - Original Tango format CAD files (ASCII).

SGLD1.NET - Tango format netlist (ASCII).

Notes:

1. A regular 2.5 V zener diode may be substituted for the TL431

regulator using the 'A' pad of the TL431 for its anode and the

separate 'K' pad for its cathode.

2. The pattern for R5, the power adjust potentiomenter, can accomodate

a variety of mounting configurations.  It is recommended that the

particular pot being used should be inserted such that its resistance

will decrease (and laser diode beam power will thus increase) with

clockwise rotation.

3. The pattern for the laser diode connector (LD1) can accomodate both

100 mil and 2 mm pin spacing.  Alternatively, the leads of the laser

diode or its extension cable may be soldered directly to the pads.

4. Take care to insert the correct pins (E, B, C) of your transistors

into the marked holes as they may not correspond to the particular

case style used.

5. R9 may be replaced with a jumper if current monitoring is not required.

6. See additional comments in the laser document and SG-LD1 schematic.